| MA700后机身项目经验与教训记录表 Record Form of MA700 Rear Fuselage Project Experience and Lessons |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 序号 Order number | 设计阶段 Design Stage | 专业 Discipline | 编写人 Prepared by | 日期 Date | 问题描述 Description | 解决情况 Resolution | 经验与教训 Experience and Lessons | 支持文件名称 Supporting file name | 遗留问题 Remaining problems | 备注 Remarks |
| 1 (1) | JCDP (2) | 结构 Structure (3) | 张三 Zhang San (4) | YYYY.MM.DD (5) | 打开驱动机构选择:气弹簧VS扭力管 The selection of open drive mechanism: gas spring VS torque tube (6) | 本项目初步采用气弹簧 Gas spring is preliminarily adopted in this project (7) | 气弹簧和扭力管均可作为打开驱动动力装置,气弹簧需要空间较小、技术相对成熟，已经有飞机采用（如B737的应急门），成本也较低，但3-5年需进行更换，维护成本较高；扭力管需占用较大的空间，目前尚没有飞机采用作为舱门的打开动力源，风险较大，但其不需要进行更换，可与机体同寿命，降低了直接维护成本。 Both the gas spring and the torque tube can be used as the open driving power device. The gas spring requires less space and maturer technology. It has been used by aircraft (such as the emergency door of B737), and the cost is lower. However, it needs to be replaced in 3-5 years, so the maintenance cost is higher; The torque tube requires a large space. At present, there is no aircraft that uses the torque tube as the power source for opening the door, which is risky, but it does not need to be replaced. It can have the same life as the aircraft, reducing the direct maintenance cost.(8) | 扭力杆报告 Report of Torque Tube (9) | 等待张三出差解决 Waiting for Zhang San's business trip to solve it. (10) | 若有不明事宜联系张三解决 If there is any unknown matter, contact Zhang San to solve it (11) |
| 2 | JDP | 强度 Strength | 李四 Li Si | YYYY.MM.DD | 舱门挡块的分布是否合理会直接影响到整体结构的受力和变形 The distribution rationality of the door block will directly affect the stress and deformation of the overall structure | 6个完整挡块+4个破损挡块 6 complete blocks and 4 broken blocks | 门挡在舱门的受力过程中起到支撑整个结构的作用，挡块的位置相当于是约束点的位置，因此会直接影响到门的整体受力变形。不合理的布置会导致个别门挡受到反向的支持力，这种不真实的结果在设计挡块布局时必须要避免，同时考虑减小门挡的受力来优化挡块数量和布局。 The door block plays a role in supporting the whole structure during the force bearing process of the door. The position of the block is equivalent to the position of the constraint point, so it will directly affect the overall force bearing deformation of the door. Unreasonable layout will lead to reverse support force on individual door blocks. This unrealistic result must be avoided when designing the block layout. At the same time, consider reducing the force on door blocks to optimize the number and layout of blocks. | 应急门初始强度设计准则 Design Criteria for Initial Strength of Emergency Door | 实施试验验证 Implementation of test verification | 若条件改变，重新准备试验 If the conditions change, prepare the test again |
| 3 | DDP | 系统 System | 丁一 Ding Yi | YYYY.MM.DD | 应急滑梯触发选择：机械触发VS电子触发 The section of emergency slide trigger: mechanical trigger VS electronic trigger | 本项目初步采用机械触发 Mechanical trigger is preliminarily adopted in this project | 机械触发和电子触发均可作为应急滑梯触发方式，机械触发相对重量较轻、技术相对成熟，已经有飞机采用（如A320）；而电子触发重量较重、成本相对较高。 Both mechanical trigger and electronic trigger can be used as emergency slide trigger. Mechanical trigger is lighter and maturer in technology, which has been used by aircraft (such as A320); The electronic trigger is heavier and more expensive. | 无 None | 等待继续评估结果 Awaiting for the results of continuing evaluation | 无 None |
| 4 | DDP | 强度 Strength | 王二 Wang Er | YYYY.MM.DD | 大窗户方案的问题 Problems of the large window scheme | 已经对窗框结构的疲劳问题进行了分析 The fatigue problem of window frame structure has been analyzed | 在有限的空间内选择大窗方案是一个风险较大的抉择，大窗不仅增加了机构布置的难度，也增加了产品装配的难度，对于窗框及周边结构的影响也需要做详细的静强度以及疲劳损伤容限分析。 It is a risky choice to choose a large window scheme in a limited space. Large windows not only increase the difficulty of mechanism layout, but also increase the difficulty of product assembly. Detailed static strength and fatigue damage tolerance analysis is also required for the impact of window frames and surrounding structures. | 无 None | 继续评估窗框的损伤容限 Continue to evaluate the damage tolerance of window frame | 无 None |

填写说明：Instructions for Filling

- 填写序号Fill in the Order number

- 填写设计阶段Fill in the Design Stage

- 填写专业Fill in the Discipline

- 填写编写人Fill in the Prepared by

- 填写日期Fill in the Date

- 填写问题描述Fill in the description

- 填写解决情况Fill in the solution

- 填写经验教训Fill in the Experience and Lessons

- 填写支持文件名称Fill in the Supporting file name

- 填写遗留问题Fill in the Remaining problems

- 填写备注Fill in the Remarks

(1)-(11)由工程项目人员填写。(1) - (11) Filled by Engineering Project.