前    言

本制度属于支撑类管理制度。

本制度由科技创新部提出，科技创新部起草。

本制度由科技创新部负责解释。

商飞项目过程流程图的编制管理标准

1  范围

本文件规定了商飞项目飞机部件过程流程图的编制和管理要求。

本文件适用于商飞项目生产。

本文件依据GLC140411和SR0014的要求编制。

2  规范性引用文件

下列文件对于本文件的应用是必不可少的。凡是注日期或（和）版次的引用文件，仅注日期或（和）版次的版本适用于本文件。凡是不注日期或（和）版次的引用文件，其最新版本（包括所有的修改单）适用于本文件。

SR0014  装配和制造工艺文件的通用规定

GLC140411      PFMEA的使用管理程序

GLC231007  技术文件的编写管理程序

GLC231008  质量记录管理程序

GLB231007-01  技术文件的编写标准

3  表格

3.1规定表格

无

3.2引用表格

无

4  术语和定义

4.1 过程流程图

对产品生产过程的展示，涵盖接收原料到交付完工产品的过程。同时也应该考虑产品在内部从一步到下一步的转移，以及内外部之间的转移。它还包括替代的过程，即用于达到相同输出的不同过程（例如：备份设备、备选供方、工序更改）。过程流程图需要每一步足够的细节来清晰和完整地描述制造产品所需的过程。

5  规定

5.1 使用过程流程图对关键/重要特性形成的工艺流程进行分析。过程流程图分析形成特性的制造（含装配、试验）、移动、存储和检验流程（步骤），明确每个步骤的功能；针对每个步骤，识别其输入特性、输出特性和过程参数。每个关键/重要特性均应有对应的过程流程图，过程流程图表单号为SAMC 03-179，具体格式及填写要求见附录A。

5.2 过程流程图应体现零部件生产过程涉及的加工、移动、检验、产品特性的输入输出等信息。复材部段过程流程图的识别分为零件及装配两个专业方面开展，复材零件专业依据工艺鉴定后制造计划的批准情况开展。

5.3 过程流程图采用纸质版审签及发放。

5.4 生产单位技术室工艺员负责过程流程图的编制、实施和更改。

5.5 过程流程图的编写、编号、版次管理和更改要求按GLC231007中的规定执行。

5.6 批产产品的过程流程图应进行生产批准。

5.7 审签流程

过程流程图纸质版审签及发放，审签、批准规定如下：

a) 生产单位技术室工艺员编制；

b) 生产单位技术室同专业工艺员校对；

c) 生产单位技术室主任审核；

d) 生产单位技术副厂长批准。

5.8 过程流程图的更改

5.8.1 应定期审查和更新过程流程图，以获取生产过程和检验的变更以及生产过程中获得的新知识，例如，从生产停止或延迟、不合格、产品质量逃逸、检验数据、对当前或类似产品的根本原因纠正措施调查中获得的教训及报废数据等，过程流程图应每年更新一次，可按实际情况缩短更新周期。

5.8.2 当任何影响产品、制造过程、质量指标等更改发生时，由生产单位技术室组织相关人员对过程流程图重新审查。

5.8.3 过程流程图的更改按GLB231007-01规定执行，更改后由生产单位技术室工艺员重新审批、发放。

6  标准图例说明

无

7  记录控制

相关记录的填写和维护按程序文件GLC231008要求执行。

附录A

（规范性）

SAMC 03-179（2024-09）填写说明

SAMC 03-179(2024-09) instructions

SAMC 03-179保存部门和保存期限：编制部门，10年。

SAMC 03-179 preservation department and

preservation period: preparation department, 10 years.

归档性质：纸质归档

Filing Nature: Paper Filing.

（1）版次：过程流程图的版次，形式如001、002、003....。必填。

Revision: Revision of process flow chart, in the form of

001, 002, 003 .... Required.

（2）过程流程图编号。必填。装配专业过程流程图编号格式为：机型-PFD-工位-顺序号，顺序号形式如001,002....，如：C919-PFD-200-001。零件专业过程流程图编号格式为：机型-PFD-零件号，如ARJ21-PFD-227A3220-002-001；当一个零件有多个过程流程图时，在零件号后面增加后缀来区别，如：ARJ21-PFD-227A3220-001-001-1。

Process flow chart number.

Required. The number format of the assembly professional process flow chart is:

model-PFD-station-sequence number, sequence number form is 001,002..., such as

C919-PFD-200-001. Part professional process flow chart number format is: model

code-PFD-part number, such as ARJ21-PFD-227A3220-002-001; When a part has

multiple process flow charts, add a suffix after the part

number to distinguish

it, such as ARJ21-PFD-227A3220-001-001-1.

（3）机型：使用该过程流程图的飞机型号。必填。

Model: The aircraft model using the process flow chart.

Required.

（4）名称/描述：该过程流程图所做的工作的描述。必填。

Name /Description: A description of the work done by the

process flowchart. Required.

（5）研制/批产：此份过程流程图应用于飞机的阶段，在研制或批产后的打√。必填。

This process flow chart is applied to the stage of the

aircraft and is ticted after development or batch. Required.

（6）编制人：过程流程图的编制人员。必填。

Prepared by: The preparer of the process flow chart.

Required.

（7）编制日期：过程流程图的编制日期。必填。

Date of preparation: The date of preparation of the

process flowchart. Required.

（8）过程编号：操作的顺序号，形式如01，02，03....。必填。

Process number: the sequence number of the operation, in

the form of 01,02,03 .... Required.

（9）制造、移动、储存、检验：操作的类型，在对应的格子内填入对应的符号“◇”、“〇”、“△”、“□”。必填。注意检验、搬运、存储等操作不能遗漏。

Manufacturing, moving, storing and inspecting: the type

of operation, fill in the corresponding symbols in the corresponding grid“◇”、“〇”、“△”、“□”. Required.

Note that inspection,

handling, storage and other operations cannot be omitted.

（10）操作描述：简述该步操作要做的工作。必填。如，产品起吊可分为“连接起吊工装”、“起吊”、“上架”、“与型架/支撑工装连接”、“拆除起吊工装”等操作。

Operation Description: Briefly describe the work to be

done in this step. Required. For example, product lifting can be divided into

"connecting lifting tooling", "lifting",

"shelving", "connecting with profile frame /support

tooling", "removing lifting tooling" and other operations.

（11）功能：该步操作的作用。必填。

Function: The function of this step operation. Required.

（12）特性编号：本步骤涉及工程规定的关键/重要特性编号，如Z101、G102等。

Characteristic number: the key characteristic number

specified by the project, such as Z101, G102, etc.

（13）产品特性（输入）：这一步操作中需要关注的产品（原材料、半成品、成品）本身的特性、本工序没开工前所需掌握的设计要求、材料特性、工装使用要求等。如，“连接起吊工装”时，需关注的产品特性有：尺寸、重量、重心、起吊工装的连接形式等。必填。如涉及关键/重要特性需注明特性编号如孔径10±0.2（Z001）。

Product characteristics (input): the characteristics of

the products (raw materials, semi-finished products and finished products) that

need to be paid attention to in this step of operation, the design

requirements, material characteristics and tooling requirements that need to be

mastered before the process starts. For example, when "connecting lifting

tooling", the product characteristics that need to be paid attention to

are: size, weight, center of gravity, connection form of lifting tooling, etc.

Required. If key /important characteristics are involved, indicate the

characteristic number such as aperture 10±0.2(Z001).

（14）过程特性（输入）：针对该步骤中所涉及的人、机、料、环的要素进行分析，如，“连接起吊工装”时，温度、湿度、起吊工装/设备的各项参数、人员要求、周围是否有障碍物等。必填。当某要素不涉及时，应该要素后注明NA，如人：NA。

Process characteristics (input): analyze the elements of

people, machines, materials and rings involved in this step, such as

temperature, humidity, various parameters of lifting tooling /equipment,

personnel requirements, whether there are obstacles around when "connecting

lifting tooling". Required. When an element is not involved, NA should be

indicated after the element, such as person: NA.

（15）产品特性（输出）：这步操作完成后需要关注的产品（原材料、半成品、成品）本身的特性，即工序完成后产品与图纸、工艺要求间的符合性关键要素。如，制孔完成后，孔的位置、直径、垂直

度、粗糙度、毛刺、零件有无变形、相邻零件是否受影响等。必填，可以与产品特性（输入）相同。如涉及关键/重要特性需在注明特性编号。如孔径10±0.2（Z001）。

Product characteristics (output): the characteristics of

the products (raw materials, semi-finished products and finished products) that

need to be paid attention to after the completion of this operation, i.e. the

key elements of compliance between the products and drawings and process

requirements after the completion of the process. For example, after the hole

is made, the position, diameter, verticality, roughness, burr, whether the

parts are deformed, whether the adjacent parts are affected, etc. Required, can

be the same as product characteristics (input). If key /important features are

involved, please indicate the feature number. Such as aperture 10±0.2(Z001).

（16）编制：编制人员签审姓名和日期。

Establishment: Establishment personnel sign and review.

（17）校对：校对人员签审姓名和日期。

Check: Check personnel sign and review.

（18）审核：审核人员签审姓名和日期。

Audit: Audit personnel sign for review.

（19）批准：批准人员签审姓名和日期。

Approval: Approval personnel sign for review.

（20）第_页共_页：对于过程流程图，填写本过程流程图的页次数和总页数；

PAGE_OF_: for paper revision Process flow chart,fill in the page number

and total page of Process flow chart.