| 编号 No | 日期 Date |  | 炉号 Furn No. | 系列号 Ser.No. |  |  | 铆钉热处理生产和检验记录 第 页共（4）页 Production and Inspection Record forRivet Heat Treatment Page of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | （1） |  | /（2） |  | /（3） |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 零件号 Part No. |  | 零件质量编号 Batch No of parts |  |  |  | 材料炉批号 Batch No. of material |  | 材料牌号 Material designation |  | 原始状态 Original Condition |  | 最终状态 Final Condition |  |  | 数量 Quantity | 规格 Size |  |  | 抽样测试样本Sample size |  | 备注 Remarks |
| （5） |  | （6） |  |  |  | （7） |  | （8） |  | （9） |  | （10） |  |  | （11） | （12） |  |  | （13） |  | （14） |
| □ 清洗（15） 操作者：（16） 检验员：（17） Cleaning Operator: Inspector: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| □ 铆钉装挂（18） 操作者：（19） 检验员：（20） Loading Operator: Inspector: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| □ 固溶处理（21） 设定温度（℃）与时间（分钟）：（22） 实际的温度（℃）与时间（分钟）：（23） Solution Required temperature(℃)/Time(min)： Actual temperature(℃)/Time(min)： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| □淬火（24） Quenching |  | 淬火转移时间(s) Time before quenching(s) |  |  |  | 淬火介质前后温度（℃） Temperature(℃)of quenching media |  |  | 干燥时间（s） Drying time(s) |  | 激冷开始 Beginning of deep cold |  |  | 激冷结束 End of deep cold |  |  |  | 铆钉冷藏 Cooling storage |  |  |  |
|  |  | （25） |  |  |  | 前Before： （26） 后After： （27） |  |  | （28） |  | 时间(min) Time:（29） 温度(℃) Temp.:（30） |  |  | 时间(min) Time : （31） 温度（℃）Temp.:（32） |  |  |  | 时间(min) Time : （33） 温度（℃）Temp.:（34） |  |  |  |
| 备注： （35） Remarks: |  |  |  |  |  |  |  |  |  |  | 操作者 Operator: |  | （36） |  |  |  | 检验员 Inspector: |  |  | （37） |  |

填写说明 Instructions for Filling：

（1）热处理工填写生产日期。

Heat treatment worker fill in the production date.

（2）热处理工填写炉子编号。

Heat treatment worker fill in the furnace number.

（3）热处理工填写系列号即生产日期内的炉批号。

Heat treatment worker shall fill in the series number, that is, the batch number within the production date.

（4）铆钉热处理生产和检验记录页数。

The number of pages of rivets' heat treatment production and inspection records.

（5）热处理工按照产品制造记录填写零件号。

The heat treatment worker shall fill in the part number according to the product manufacturing record.

（6）热处理工按照产品制造记录填写质量编号。

The heat treatment worker shall fill in the quality number according to the product manufacturing record.

（7）热处理工按照产品制造记录填写材料炉批号。

The heat treatment worker shall fill in the batch number of the material furnace according to the product manufacturing record.

（8）热处理工按照产品制造记录填写材料牌号。

The heat treatment worker shall fill in the material designation according to the product manufacturing record.

（9）热处理工按照产品制造记录填写材料原始状态。

The heat treatment worker shall fill in the original state of the material according to the product manufacturing records.

（10）热处理工按照产品制造记录填写材料热处理最终状态。

The heat treatment worker shall fill in the final state of the material heat treatment according to the product manufacturing records.

（11）热处理工按照产品制造记录填写零件数量。

Heat treatment workers fill in the number of parts according to the product manufacturing records.

（12）热处理工按照产品制造记录填写材料规格。

The heat treatment worker shall fill in the material specifications according to the product manufacturing records.

（13）热处理工按照典规选取抽样测试样本。

The heat treatment worker selects sampling test samples according to the Typical Process Instruction.

（14）热处理过程中的特殊情况需做备注。

Remarks should be made for special situations in the heat treatment process.

（15）热处理工按照热处理典规判断铆钉是否需要清洗，若需要清洗则在□打“√”，若不适用划“/”。

The heat treatment worker shall judge whether the rivets need to be cleaned according to the Typical Process Instruction of heat treatment. If they need to be cleaned, they shall be cleaned in □Mark √ or / if not applicable.

（16）操作者（热处理工）盖章并注明日期。

Stamped and dated by the operator (heat treatment worker).

（17）检验员盖章并注明日期。

Stamped and dated by the inspector.

（18）热处理工按照热处理典规判断铆钉是否需要装挂，若需要装挂则在□打“√”，若不适用划“/”。

The heat treatment worker shall determine whether the rivets need to be mounted and hung according to the Typical Process Instruction of heat treatment. If so, the rivets shall be mounted and hung in □Mark √ or / if not applicable.

（19）操作者（热处理工）盖章并注明日期。

Stamped and dated by the operator (heat treatment worker).

（20）检验员盖章并注明日期。

Stamped and dated by the inspector.

（21）热处理工按照热处理典规判断铆钉是否需要固溶处理，若需要固溶处理则在□打“√”，若不适用划“/”。

The heat treatment worker shall determine whether the rivets need solid solution treatment according to the Typical Process Instruction of heat treatment, and if so, the solution place Mark "√" on □, or "/" if not applicable.

（22）热处理工按照热处理典规填写设定温度与要求的保温时间。

The heat treatment worker shall fill in the set temperature and the required holding time according to the Typical Process Instruction of heat treatment.

（23）热处理工按照热处理温度曲线填写实际温度与实际的保温时间。

Heat treatment workers fill in the actual temperature and actual holding time according to the heat treatment temperature curve.

（24）热处理工按照热处理典规判断铆钉是否需要淬火，若需要淬火则在□打“√”，若不适用划“/”。

The heat treatment worker shall judge whether the rivet needs quenching according to the Typical Process Instruction of heat treatment. If it needs quenching, it shall be done in □Mark √, if not applicable mark /.

（25）热处理工填写实测的淬火转移时间。

The heat treatment worker fills in the measured quenching transfer time.

（26）热处理工填写淬火介质淬火前的温度。

Temperature of quenching medium filled by heat treatment worker before quenching.

（27）热处理工填写淬火介质淬火后的温度。

The temperature after the quenching medium is filled by the heat treatment worker.

（28）热处理工填写干燥时间。

Heat treatment worker fill in the drying time.

（29）热处理工填写激冷开始时间。

Heat treatment worker fill in the starting time of chilling.

（30）热处理工填写激冷开始温度。

Heat treatment worker fill in the starting temperature of chilling.

（31）热处理工填写激冷结束时间。

Heat treatment worker fill in the end time of deep cold.

（32）热处理工填写激冷结束温度。

Heat treatment worker fill in the end temperature of deep cold.

（33）热处理工填写铆钉冷藏时间。

Heat treatment worker fill in the rivet refrigeration time.

（34）热处理工填写铆钉冷藏温度。

Heat treatment worker fill in the rivet refrigeration temperature.

（35）针对热处理过程中的特殊情况需做备注。

Remarks should be made for special conditions in the heat treatment process.

（36）操作者（热处理工）盖章并注明日期。

Stamped and dated by operator (heat treatment worker).

（37）检验员盖章并注明日期。

Stamped and dated by the inspector.