| 机型Model |  | 制 造 大 纲 Fabrication Outline | 零件号P/N |  | 厂代号Shop No. |  | 第 (5) 页Page (5) |  | 下转第 (6) 页Next Page (6) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) |  |  | (2) |  | (3) |  |  |  |  |  |
| 预检 PI | 工序号 Op.No | 工 作 说 明 Work Description |  | 工 装 Tools |  | 设 备 Equip |  | 操 作 者Operator | 数量Q/Y | 检验员Inspector |
| (50) | (20) | (21) |  | (22) |  | (23) |  | (45) | (47) | (48) |

填写说明Instructions for Filling：

(1) 填写机型或项目标记号。

Fill in the model or item tag number.

(2) 填写按本制造大纲生产的零/组件图号,或工艺构型号(试验件前加“S-”)。如工程中零/组件有版次（如更改字母等）记录要求,则需加入该版次等符号。

Fill in the drawing number of the parts/components produced according to this Fabrication Outline, or the model of the process structure (add "S -" before the test piece). If the parts/components in the project have revision (such as letter change) record requirements, the revision and other symbols shall be added.

(3) 填写生产厂代号。

Fill in the workshop code.

(5) 填写制造大纲的页次，从封页开使按自然数1、2、3...顺序编排。如有加页按1a，1b，1c等等填写。

Fill in the page number of the Fabrication Outline, and arrange it in the order of natural number 1, 2, 3 from the cover page. If there are additional pages, please fill in 1a, 1b, 1c, etc.

(6) 填写下一页的页次号，最后一页填“X”。

Fill in the page number on the next page, and fill in "X" on the last page.

(20) 填写本工序的顺序号，原则上按10、15、20……顺序填写。当增加工序时，可使用中间的空号。

Fill in the sequence number of this process in the order of 10, 15, 20... in principle. When adding an operation, you can use the vacant number in the middle.

注:如因工装、设备等原因,需将一道工序分成几个工步进行时,按“工序号－工步号(自然数)”填写。

Note: If a process needs to be divided into several steps due to tooling, equipment and other reasons, fill in according to "Process No. - Step No. (natural number)".

(21) 填写工序内容说明。

Fill in the process description.

(22) 填写本工序使用的专用工装、工具和刀量具的名称和图号。有版次要求的专用工装应注明其版次号。版次要用“”引上, 如“T”。

Fill in the name and drawing number of special tooling, tools, knives and measuring tools used in this process. Special tooling with version requirements shall be marked with its version number. The version shall be introduced with "", such as "T".

(23) 填写本工序所用设备的名称，专用设备要注明型号。

Fill in the name of the equipment used in this process, and indicate the model of the special equipment.

(45) 该工序完成后，由操作者在制造大纲原件上盖质量印章/签字，并注明日期；当零/组件在各生产厂或检测单位移交时，需按要求进行。

After the process is completed, the operator shall affix the quality seal/signature on the original Fabrication Outline and indicate the date; When parts/components are handed over in each manufacturer or testing unit, they shall be handed over as required.

(47)、(48) 检验员在检验工序或检查控制点验收合格后，填写验收数(只填写数字，不写单位)盖检验印章/签字，并注明日期。当发生不符合时，按GLC14080401做PI记录。

After the inspection process or inspection control point is accepted, the inspector shall fill in the acceptance amount (only fill in the number, not the unit), affix the inspection seal/signature, and indicate the date. In case of nonconformity, PI records shall be made as per GLC14080401.

(50) 由检验部门按GLC14080401填写。

The inspection department shall fill in according to GLC14080401.