Foreword

This system is an operational management regulation.Proposed by the Department of Science and Technology Innovation, this system is drafted with the participation of the Department of Science and Technology Innovation.The Department of Science and Technology Innovation is responsible for the interpretation of this system.

Rules for the Management of Tooling and Tools

1  Scope

This document specifies the establishment of processes to sort out the full-life-cycle control plan for tooling and tools.This document applies to civil aircraft projects/all projects.These rules are formulated in accordance with GS14 Integrated Manufacturing Management Manual.

2  Normative References

The following documents are indispensable for the application of this document. For dated and/or versioned referenced documents, only the edition with the indicated date and/or version applies to this document. For undated and/or unversioned referenced documents, the latest edition (including all amendments) applies to this document.

AS/EN9100 Quality Management Systems — Requirements for Aviation, Space and Defense Organizations

GS00 AOS Management Manual

GS03 Civil Aircraft Quality Manual

3  Terms and Definitions

None

4  Organizations and Responsibilities

4.1 Department of Science and Technology Innovation

As the centralized management department for tooling and tools, the Department of Science and Technology Innovation has the following main responsibilities:a) Responsible for the overall management of the company's tooling and tool business;b) Responsible for tooling application approval, tooling design, submission of external procurement applications, organizing tooling manufacturing, tooling acceptance, regular tooling inspection, tooling repair implementation, and monitoring and management of the daily use and maintenance of tooling;c) Responsible for the review and approval of tool applications, technical appraisal of tool maintenance and scrapping, and daily management.

4.2 Tooling User Units

Tooling user units are responsible for the application, use, and maintenance management of tooling and tools, with the following main responsibilities:a) Responsible for the application for types of tooling used by the unit, repair applications, tooling acceptance, regular tooling inspection, daily use and maintenance of tooling, asset management, and tooling status control;b) Responsible for the application, receipt, inspection, registration, use, borrowing, and daily management of tooling and tools within the unit.

4.3 Production and Operation Department

a) Responsible for the implementation of tool procurement and tracking the status of tool procurement.

4.4 Quality Management Department

a) Responsible for the verification and maintenance of measuring tools and coordinating the resolution of related issues.

5  Management Requirements

5.1 Planning Requirements for Tooling and Tools

a) In accordance with the main phase plan and overall manufacturing plan requirements, plan the tooling and tools required for aircraft product realization, determine the types and quantities of tooling and tools needed, and formulate procurement budgets;b) Compile the list of tooling and tool types in accordance with the overall manufacturing plan requirements, as well as tooling calculation sheets, tooling selection coefficients and control numbers, interchangeability/replaceability plans, assembly coordination plans, and tool procurement principles;c) Based on the regular tooling inspection catalog, the implementation status of regular tooling inspections, and the production schedule of tooling user units, carry out planning for the implementation of regular tooling inspections and issue the Regular Tooling Inspection Schedule.

5.2 Requirements for Tooling Application and Manufacturing

a) Tooling Application Management

Submit tooling applications in accordance with the approved list of tooling types or change orders to the list of tooling types.

Standard forms shall be used for tooling applications, which shall include at least the tooling name, tooling number, and quantity.

b) Tooling Design and Modification

Tooling design shall comply with the tooling production plan, tooling design technical conditions, engineering documents or data, and other design bases, and meet the requirements of design and development planning.

Output management of design and development shall be carried out for tooling design to ensure a reasonable tooling structure, meeting the requirements of tooling use, tooling manufacturing, safety, and other product realization requirements.

The selection of tooling materials and finished products shall consider costs, and standard parts and finished parts shall be used as much as possible to improve the tooling standardization coefficient. Tooling design shall comply with national standards, industry standards, enterprise standards, and special customer requirements, meeting the requirements of design and development process control.

Tooling drawings can only be officially released after being checked, reviewed, and approved. When necessary, they shall be subject to standardization review and countersigned by the user unit and customer to ensure compliance with design and development change management requirements.

Tooling design is a low-risk development process.

For inspection tooling, standard tooling, assembly tooling, spreaders, debris removal equipment, product cleaning equipment, and test equipment, tooling certification documents shall be issued indicating the tooling name, number, status, etc.

Tooling shall be designed in-house, with external design for insufficient resources, and tooling manufacturing shall be provided by external resources.

Storage requirements for stored tooling shall be specified, including regular protection/status inspections.

c) Management of Tooling Data

Catalogs, temporary storage areas, and release areas shall be established in the computer system, and the catalogs shall indicate the version validity of the data sets. Permission control for storage and access to the storage areas and release areas shall be clearly defined.

Updates to data sets shall be declared when released in the computer system.

Data sets shall be approved before release or re-release after update.

Data sets shall be backed up regularly, and the storage of backup data shall be secured.

d) Tooling Manufacturing Management

Tooling shall be manufactured in accordance with approved process procedures, national standards, industry standards, enterprise standards, manufacturing specifications, and approved design requirements.

The characteristics of tooling drawings shall be checked to verify compliance with requirements.

When necessary, the delivery of tooling shall be accompanied by measurement data of tooling datums and proof of deviation rejection handling.

Tooling used for production process automation and control/monitoring shall be confirmed and maintained before being used in production.

Inspection work shall be carried out based on valid tooling drawings. During tooling acceptance, relevant records or quality assurance records shall be completed, and qualified certification documents shall be issued after tooling acceptance.

5.3 Operation Management of Tooling

a) Use of Tooling

When tooling is transferred from the manufacturing unit to the user unit, there shall be written documents confirming the receipt of the tooling and its completeness.

Tooling user units shall establish accounts for the received tooling for management purposes.

Operation instructions shall be compiled for complex tooling such as component assembly jigs, inspection jigs, precision machining tables, and large-scale complex test benches.

Necessary visual and functional inspections shall be carried out before the use of tooling. If any problems are found during the inspection, the use of the tooling shall be stopped or restricted, and obvious marks shall be made. At the same time, the fault shall be promptly reported to the technical office of the user unit.

The use of tooling shall ensure that the function of the tooling is not damaged.

Necessary protection shall be provided for tooling during storage and transportation. Measures shall be taken to restrict the unauthorized use and transportation of standard tooling.

Customer-supplied tooling assets shall, in principle, be owned by the customer, and specific circumstances shall be implemented in accordance with the signed project contract. The use and management of customer-supplied tooling include the receipt, storage, borrowing, maintenance, use, regular inspection, asset management, and return of customer-supplied tooling.

b) Regular Inspection of Tooling

A catalog of regular tooling inspections shall be issued for tooling requiring regular inspections, specifying the scope and cycle of regular inspections.

Technical requirement documents for regular inspections shall be issued for tooling requiring regular inspections, specifying the content of regular inspections, which shall be carried out accordingly.

Regular tooling inspection plans shall be issued in accordance with the regular inspection cycle, specifying the time or number of aircraft for regular tooling inspections, which shall be carried out as planned.

Tooling shared by production and inspection shall be calibrated or verified as qualified before being used for inspection, and records shall be retained.

The calibration of tooling shared by production and inspection before use for inspection shall ensure that the calibrated characteristics do not change during use or that the change is not sufficient to affect the product inspection process.

c) Rework of Tooling

Rework of tooling physical objects shall be carried out in accordance with the latest valid tooling drawings.

When necessary, the characteristics of the reworked tooling drawings shall be checked to verify compliance with requirements.

When necessary, the delivery of tooling shall be accompanied by measurement data of tooling datums and proof of deviation rejection handling.

d) Maintenance of Tooling

Tooling shall be regularly maintained and inspected to ensure its completeness and good condition.

When tooling is not used for a long time, it shall be sealed.

Before each start of work, tooling users shall inspect the completeness (whether the hole position bushings are firm and not missing), appearance, tightness, use function, safety measures, etc., of the tooling used.

The Tooling Room shall organize physical verification of tooling. Tooling user units shall inspect tooling for wear and damage to ensure its completeness and good condition.

Tooling user units shall use tooling correctly. If any faults or abnormalities are found, the use of the tooling shall be stopped immediately. Inspections for tooling wear and damage shall be carried out to ensure its completeness and good condition. If tooling wear or damage requires maintenance assistance from the Tooling Room, an application may be submitted. After technical appraisal by the technicians of the Tooling Room, a work order shall be issued for maintenance.

5.4 Verification of Tooling

a) Tooling verification shall be carried out before tooling is put into use. If any problems are found during the verification, the use of the tooling shall be stopped or restricted, and obvious marks shall be made. At the same time, the fault shall be promptly reported to the Tooling Room.b) Tooling verification must be carried out for newly manufactured and efficient tooling.c) For reworked/modified tooling, partial or full tooling verification shall be carried out when necessary.d) When the product characteristics of tooling used for inspection purposes cannot be directly accepted in accordance with engineering bases, the tooling must be verified through the assembly practice of at least three aircraft.

5.5 Optimization of Tooling

a) In accordance with the requirements of the main schedule and capacity improvement, combined with the tooling calculation sheet, put forward requirements for optimizing the quantity of tooling.b) For tooling scheme optimization found during tooling use, verification, and product production, put forward technical conditions for tooling optimization and carry out tooling optimization and version upgrade.

5.6 Application for Tools

a) Specify the definition of general tools, the responsibilities of each department for general tool applications, the review and approval process for general tool applications, and the standard forms for general tool applications.b) Specify the definition of special tools, the responsibilities of each department for special tool applications, the review and approval process for special tool applications, and the standard forms for special tool applications.c) Specify the work requirements for tracking information throughout the entire process from tool application, tool procurement, tool receipt to tool accounting and payment, as well as the work requirements for tool inventory.

5.7 Design and Modification of Special Tools

a) Specify the design basis for special tools, as well as the relevant requirements for the review, approval, printing, and distribution of drawings.

5.8 On-site Control of Tools

a) Specify the management requirements for the receipt and acceptance of new tools.b) Specify the scope and requirements for tool maintenance management, and clarify the responsibilities of each department in tool maintenance management.c) Tool maintenance management mainly includes cutting tool regrinding, pneumatic and electric tool maintenance, and other tool maintenance.d) Specify the specific requirements for warehouse management and clarify the job responsibilities of warehouse management.e) Specify the specific requirements for tool transfer, the responsibilities of each department for tool transfer, and the review and approval process for tool transfer orders.f) Specify the specific requirements and precautions for tool use, maintenance, and control.g) Tool maintenance management mainly includes the use, regular inspection, and scrapping appraisal of slings, as well as the relevant requirements for the maintenance of important tools.h) Specify the rules for tool scrapping, the handling of lost tools, the calculation of compensation for lost tools, and the management of scrapped tools.

5.9 Verification of Tools

a) Specify the requirements, standards, processes, and methods for tool verification.

5.10 Trial Use and Selection of Tools

a) Specify the application process for the trial use of new tools, the review and approval of new tool reports, and the feedback requirements for the small-batch use of tools.

6 Assessment, Rewards and Penalties

6.1 Execution of Tooling Budget

a) The execution rate of the new tooling manufacturing budget shall not be less than 85%;b) The execution rate of the budget for tooling regular inspection and maintenance shall not be less than 85%.

6.2 Execution of Tool Budget

a) The execution rate of the new tool purchase budget shall not be less than 85%;b) The execution rate of the tool maintenance cost budget shall not be less than 85%.

Appendix A

(Normative)

None